CargoElixir Fleet Manager

Elixir Developer Implementation Plan

MQTT Fleet Management, Gateway Provisioning, and Remote Control

March 2026  |  Buoy.fish Engineering

This document targets an Elixir developer joining the CargoElixir codebase. It assumes comfort with Phoenix, OTP, and Ecto but not necessarily LoRaWAN.

# 1. Context and Goals

This plan adds gateway fleet management to the existing CargoElixir Phoenix application (app.buoy.fish). The system manages a fleet of Raspberry Pi companion devices deployed alongside LoRaWAN gateways in remote fishing communities (Baja Mexico, Nova Scotia). Each Pi runs Docker containers for LoRa packet forwarding and a lightweight Python agent that connects to the Phoenix server via MQTT.

## 1.1 What Already Exists

Application: CargoElixir (Phoenix 1.7.14 + React 18.2 frontend). Not LiveView. The React app communicates via REST API and WebSocket channels.

ChirpStack Integration: Full gRPC client (CargoElixir.Chirpstack.GrpcClient) with protobuf definitions, dual-mode routing (gRPC/REST via CHIRPSTACK_USE_GRPC flag), and batch operation support.

Gateway CRUD: CargoElixirWeb.GatewayController with index, show, create, update, delete, sync_status, sync_all, and list_chirpstack_gateways. Gateway schema has role (standard/border/relay), state (never_seen/online/offline), chirpstack_synced, and location fields.

WebSocket Channels: CargoElixirWeb.UserSocket with Phoenix.Token auth (24h max_age). PayloadChannel joins on "payload:org:{slug}" with org membership verification.

Auth: Supabase JWT + JWKS verification. Auth pipeline in router checks membership, admin, owner, and platform_admin roles.

Infrastructure: Mosquitto MQTT broker at mqtt.buoy.fish:1883 (auth: gateway/buoy). Three gateways registered in ChirpStack (two border, one relay). Balena-based Pi image (being replaced) with UDP PF, ChirpStack Multiplexer, and Helium gateway-rs.

## 1.2 What This Plan Adds

| Feature | Description |
| --- | --- |
| MQTT Subscriber | GenServer in Phoenix supervision tree that subscribes to fleet heartbeats, logs, and command acks from Pi agents |
| Fleet API Endpoints | REST API endpoints consumed by the React frontend for device status, command dispatch, and log streaming |
| Fleet WebSocket Channel | New "fleet:org:{slug}" channel following the PayloadChannel pattern for real-time device status and log streaming to React |
| Remote Commands | Restart containers, reboot devices, reroute LNS endpoints, pull logs from any Pi |
| Gateway Provisioning API | Generate site configs (site.json) for new Pi companions via the existing admin API |

# 2. Architecture Overview

The fleet management plane has three layers. The key architectural decision is that the React frontend consumes REST endpoints for initial data and WebSocket channels for real-time updates (matching the existing PayloadChannel pattern), not LiveView.

| Layer | Technology | Responsibility |
| --- | --- | --- |
| Transport | MQTT (mqtt.buoy.fish:1883) | Pub/sub messaging between Pi agents and Phoenix server |
| Application | Phoenix GenServer + REST API + WebSocket Channel | Subscribe to fleet topics, process heartbeats, dispatch commands, expose endpoints to React |
| Network | Tailscale VPN | Direct SSH access to any Pi. Managed via Tailscale admin console, not by Phoenix. |

## 2.1 MQTT Topic Structure

All fleet MQTT traffic uses the buoy/fleet/ prefix, separate from the ChirpStack LoRaWAN traffic on us915_1/. The {id} is the device_id from the Pi's site.json (e.g., baja-companion-1).

| Topic | Direction | Payload (JSON) |
| --- | --- | --- |
| buoy/fleet/{id}/status | Pi -> Server | {device_id, site_name, timestamp, tailscale_ip, system: {cpu_percent, memory_percent, cpu_temp, disk_percent, uptime_seconds}, containers: [{name, status, health, image}]} |
| buoy/fleet/{id}/logs | Pi -> Server | {source, timestamp, lines: [string]} |
| buoy/fleet/{id}/commands | Server -> Pi | {action, params, request_id} |
| buoy/fleet/{id}/commands/ack | Pi -> Server | {request_id, status, result, timestamp} |
| buoy/fleet/{id}/config | Server -> Pi | Full or partial config object (agent applies and restarts services) |

## 2.2 Agent Commands

The Python agent on each Pi accepts these commands via the commands topic:

| Action | Params | Description |
| --- | --- | --- |
| restart_container | {name: string} | Restart a specific Docker container by name |
| restart_all | {} | docker compose restart on the entire stack |
| reboot | {} | Reboot the Pi (5s delay) |
| reroute_lns | {server: string, uplink_only: bool} | Add new LNS endpoint to multiplexer. This is the insurance feature. |
| update_mux_config | {config: string} | Replace entire multiplexer TOML config |
| get_status | {} | Force an immediate heartbeat response |
| get_logs | {name: string, lines: int} | Get last N log lines from a container |

# 3. Elixir Implementation

## 3.1 New Dependencies

Only one new dependency is needed. The app already has jason, phoenix_pubsub, and ecto:

# In mix.exs, add to deps:

{:tortoise311, "~> 0.12"}   # MQTT client (maintained fork of Tortoise)

Note: Do NOT add {:req, ...} or build a new ChirpStack REST client. The app already has a complete gRPC client at CargoElixir.Chirpstack.GrpcClient with gateway CRUD and batch operations.

## 3.2 MQTT Connection (GenServer)

Create a GenServer that maintains a persistent MQTT connection and routes incoming messages to the appropriate handlers. Add this to the supervision tree in CargoElixir.Application.

Module: CargoElixir.Fleet.MqttClient

defmodule CargoElixir.Fleet.MqttClient do

use GenServer

require Logger

@broker Application.compile_env(:cargo_elixir, [:fleet, :mqtt_broker], "mqtt.buoy.fish")

@port Application.compile_env(:cargo_elixir, [:fleet, :mqtt_port], 1883)

@client_id "phoenix-fleet-manager"

@subscribe_topics [

{"buoy/fleet/+/status", 1},

{"buoy/fleet/+/logs", 0},

{"buoy/fleet/+/commands/ack", 1}

]

def start_link(opts) do

GenServer.start_link(__MODULE__, opts, name: __MODULE__)

end

# Public API

def send_command(device_id, action, params \\ %{}) do

GenServer.call(__MODULE__, {:send_command, device_id, action, params})

end

def send_config(device_id, config) do

GenServer.call(__MODULE__, {:send_config, device_id, config})

end

# GenServer callbacks

@impl true

def init(_opts) do

mqtt_user = Application.get_env(:cargo_elixir, [:fleet, :mqtt_user], "gateway")

mqtt_pass = Application.get_env(:cargo_elixir, [:fleet, :mqtt_pass], "buoy")

{:ok, _conn} = Tortoise311.Connection.start_link(

client_id: @client_id,

server: {Tortoise311.Transport.Tcp, host: @broker, port: @port},

user_name: mqtt_user,

password: mqtt_pass,

handler: {CargoElixir.Fleet.MqttHandler, []},

subscriptions: @subscribe_topics

)

{:ok, %{}}

end

@impl true

def handle_call({:send_command, device_id, action, params}, _from, state) do

request_id = Ecto.UUID.generate()

payload = Jason.encode!(%{

action: action,

params: params,

request_id: request_id

})

topic = "buoy/fleet/#{device_id}/commands"

Tortoise311.publish(@client_id, topic, payload, qos: 1)

{:reply, {:ok, request_id}, state}

end

@impl true

def handle_call({:send_config, device_id, config}, _from, state) do

topic = "buoy/fleet/#{device_id}/config"

Tortoise311.publish(@client_id, topic, Jason.encode!(config), qos: 1)

{:reply, :ok, state}

end

end

Module: CargoElixir.Fleet.MqttHandler

Tortoise handler that routes incoming MQTT messages to the DeviceRegistry and broadcasts via Phoenix.PubSub (using the existing CargoElixir.PubSub instance):

defmodule CargoElixir.Fleet.MqttHandler do

use Tortoise311.Handler

require Logger

@impl true

def handle_message(["buoy", "fleet", device_id, "status"], payload, state) do

case Jason.decode(payload) do

{:ok, status} ->

CargoElixir.Fleet.DeviceRegistry.update_status(device_id, status)

# Broadcast to fleet channel (mirrors PayloadChannel pattern)

CargoElixirWeb.Endpoint.broadcast(

"fleet:status",

"device_status",

%{device_id: device_id, status: status}

)

{:error, _} ->

Logger.warning("Invalid status payload from #{device_id}")

end

{:ok, state}

end

@impl true

def handle_message(["buoy", "fleet", device_id, "logs"], payload, state) do

case Jason.decode(payload) do

{:ok, log_entry} ->

CargoElixirWeb.Endpoint.broadcast(

"fleet:logs:#{device_id}",

"log_entry",

%{device_id: device_id, entry: log_entry}

)

_ -> :ok

end

{:ok, state}

end

@impl true

def handle_message(["buoy", "fleet", device_id, "commands", "ack"], payload, state) do

case Jason.decode(payload) do

{:ok, ack} ->

CargoElixir.Fleet.CommandTracker.resolve(ack["request_id"], ack)

CargoElixirWeb.Endpoint.broadcast(

"fleet:commands:#{device_id}",

"command_ack",

%{device_id: device_id, ack: ack}

)

_ -> :ok

end

{:ok, state}

end

end

## 3.3 Device Registry (ETS-backed GenServer)

Maintains in-memory state for all fleet devices. Persists to the database periodically but serves reads from ETS for dashboard performance. This is a new module in the CargoElixir.Fleet namespace.

defmodule CargoElixir.Fleet.DeviceRegistry do

use GenServer

@table :fleet_devices

@offline_threshold_seconds 180   # 3 missed heartbeats = offline

def start_link(_), do: GenServer.start_link(__MODULE__, [], name: __MODULE__)

def init(_) do

:ets.new(@table, [:named_table, :set, :public, read_concurrency: true])

schedule_cleanup()

{:ok, %{}}

end

def update_status(device_id, status) do

record = %{

device_id: device_id,

site_name: status["site_name"],

tailscale_ip: status["tailscale_ip"],

system: status["system"],

containers: status["containers"],

last_seen: DateTime.utc_now(),

online: true

}

:ets.insert(@table, {device_id, record})

end

def list_devices do

:ets.tab2list(@table)

|> Enum.map(fn {_key, record} -> record end)

|> Enum.sort_by(& &1.site_name)

end

def get_device(device_id) do

case :ets.lookup(@table, device_id) do

[{_, record}] -> {:ok, record}

[] -> {:error, :not_found}

end

end

# Mark devices offline if no heartbeat received

def handle_info(:cleanup, state) do

threshold = DateTime.add(DateTime.utc_now(), -@offline_threshold_seconds)

:ets.foldl(fn {id, record}, _acc ->

if DateTime.before?(record.last_seen, threshold) do

:ets.insert(@table, {id, %{record | online: false}})

end

end, nil, @table)

schedule_cleanup()

{:noreply, state}

end

defp schedule_cleanup, do: Process.send_after(self(), :cleanup, 60_000)

end

## 3.4 Command Tracker

Tracks in-flight commands and resolves them when the Pi agent acknowledges:

defmodule CargoElixir.Fleet.CommandTracker do

use GenServer

@timeout_ms 30_000

def start_link(_), do: GenServer.start_link(__MODULE__, %{}, name: __MODULE__)

def init(_), do: {:ok, %{pending: %{}}}

def track(request_id, from) do

GenServer.cast(__MODULE__, {:track, request_id, from})

end

def resolve(request_id, result) do

GenServer.cast(__MODULE__, {:resolve, request_id, result})

end

def handle_cast({:track, request_id, from}, state) do

timer = Process.send_after(self(), {:timeout, request_id}, @timeout_ms)

pending = Map.put(state.pending, request_id, %{from: from, timer: timer})

{:noreply, %{state | pending: pending}}

end

def handle_cast({:resolve, request_id, result}, state) do

case Map.pop(state.pending, request_id) do

{nil, _} -> {:noreply, state}

{%{from: from, timer: timer}, pending} ->

Process.cancel_timer(timer)

if from, do: GenServer.reply(from, {:ok, result})

{:noreply, %{state | pending: pending}}

end

end

def handle_info({:timeout, request_id}, state) do

case Map.pop(state.pending, request_id) do

{nil, _} -> {:noreply, state}

{%{from: from}, pending} ->

if from, do: GenServer.reply(from, {:error, :timeout})

{:noreply, %{state | pending: pending}}

end

end

end

# 4. Fleet WebSocket Channel

Important: The app does NOT use LiveView. The React frontend connects via WebSocket channels. Follow the exact same pattern as PayloadChannel.

## 4.1 Register Channel in UserSocket

Add the fleet channel to the existing CargoElixirWeb.UserSocket:

# In lib/cargo_elixir_web/channels/user_socket.ex

# Add this line next to the existing payload channel:

channel "payload:*", CargoElixirWeb.PayloadChannel

channel "fleet:*", CargoElixirWeb.FleetChannel       # <-- ADD THIS

## 4.2 FleetChannel Module

Follows the PayloadChannel pattern: org-scoped join with membership verification, then PubSub subscription for real-time updates.

defmodule CargoElixirWeb.FleetChannel do

use Phoenix.Channel

alias CargoElixir.Accounts

# Join the fleet status feed for an organization

# Topic: "fleet:org:{slug}"

def join("fleet:org:" <> org_slug, _params, socket) do

case socket.assigns[:current_user] do

nil ->

{:error, %{reason: "unauthorized"}}

current_user ->

normalized_slug = normalize_slug(org_slug)

case Accounts.get_organization_by_slug(normalized_slug) do

nil ->

{:error, %{reason: "organization_not_found"}}

organization ->

is_member = Accounts.is_member?(organization.id, current_user.id)

is_admin = CargoElixir.Permissions.Platform.is_platform_admins?(current_user)

if is_member or is_admin do

# Subscribe to fleet PubSub topics

Phoenix.PubSub.subscribe(CargoElixir.PubSub, "fleet:status")

# Send current device list on join

devices = CargoElixir.Fleet.DeviceRegistry.list_devices()

send(self(), {:after_join, devices})

{:ok, assign(socket, :org_slug, normalized_slug)}

else

{:error, %{reason: "not_a_member"}}

end

end

end

end

# Join device-specific log stream

# Topic: "fleet:logs:{device_id}"

def join("fleet:logs:" <> device_id, _params, socket) do

case socket.assigns[:current_user] do

nil -> {:error, %{reason: "unauthorized"}}

_user ->

Phoenix.PubSub.subscribe(CargoElixir.PubSub, "fleet:logs:#{device_id}")

{:ok, assign(socket, :device_id, device_id)}

end

end

def handle_info({:after_join, devices}, socket) do

push(socket, "fleet_snapshot", %{devices: devices})

{:noreply, socket}

end

# Forward PubSub broadcasts to the WebSocket

def handle_info(%Phoenix.Socket.Broadcast{event: event, payload: payload}, socket) do

push(socket, event, payload)

{:noreply, socket}

end

# Handle commands from the React frontend

def handle_in("send_command", %{"device_id" => id, "action" => action} = params, socket) do

cmd_params = Map.get(params, "params", %{})

case CargoElixir.Fleet.MqttClient.send_command(id, action, cmd_params) do

{:ok, request_id} ->

{:reply, {:ok, %{request_id: request_id}}, socket}

{:error, reason} ->

{:reply, {:error, %{reason: reason}}, socket}

end

end

defp normalize_slug(slug) do

slug |> URI.decode() |> String.downcase() |> String.replace(~r/\s+/, "-")

end

end

## 4.3 REST API Endpoints

For initial page load and non-real-time operations, add fleet endpoints to the existing router. These go in the platform_admins scope (or create a new fleet admin scope):

# In lib/cargo_elixir_web/router.ex

# Add inside the platform_admins scope:

scope "/api/admin", CargoElixirWeb do

pipe_through [:api, :auth, :platform_admins]

# Existing gateway routes...

resources "/gateways", GatewayController, except: [:new, :edit]

# New fleet routes

get "/fleet/devices", FleetController, :index

get "/fleet/devices/:device_id", FleetController, :show

post "/fleet/devices/:device_id/command", FleetController, :send_command

post "/fleet/provision", FleetController, :generate_site_config

end

Module: CargoElixirWeb.FleetController

defmodule CargoElixirWeb.FleetController do

use CargoElixirWeb, :controller

def index(conn, _params) do

devices = CargoElixir.Fleet.DeviceRegistry.list_devices()

json(conn, %{data: devices})

end

def show(conn, %{"device_id" => device_id}) do

case CargoElixir.Fleet.DeviceRegistry.get_device(device_id) do

{:ok, device} -> json(conn, %{data: device})

{:error, :not_found} -> conn |> put_status(404) |> json(%{error: "device not found"})

end

end

def send_command(conn, %{"device_id" => id, "action" => action} = params) do

cmd_params = Map.get(params, "params", %{})

case CargoElixir.Fleet.MqttClient.send_command(id, action, cmd_params) do

{:ok, request_id} -> json(conn, %{request_id: request_id, status: "sent"})

{:error, reason} -> conn |> put_status(500) |> json(%{error: reason})

end

end

def generate_site_config(conn, %{"device_id" => device_id} = params) do

config = %{

device_id: device_id,

site_name: params["site_name"] || "",

hostname: "buoy-#{device_id}",

mqtt_broker: "mqtt.buoy.fish",

mqtt_user: "gateway",

mqtt_pass: "buoy",

tailscale_auth_key: params["tailscale_auth_key"] || "",

region: params["region"] || "US915",

gateway_eui: params["gateway_eui"] || ""

}

json(conn, %{data: config})

end

end

# 5. Supervision Tree Changes

Add the fleet processes to CargoElixir.Application. Insert them after the existing children (Repo, PubSub, GRPC.Client.Supervisor, etc.) and before the Endpoint:

# In lib/cargo_elixir/application.ex, add to children list:

# (after {Task.Supervisor, name: CargoElixir.TaskSupervisor})

CargoElixir.Fleet.DeviceRegistry,

CargoElixir.Fleet.CommandTracker,

CargoElixir.Fleet.MqttClient,

# (before CargoElixirWeb.Endpoint)

Startup order matters: DeviceRegistry must start before MqttClient, because incoming MQTT messages immediately call DeviceRegistry.update_status/2. CommandTracker must also be running before MqttClient processes acks.

## 5.1 Configuration

Add fleet config to config/runtime.exs following the existing pattern (env var with defaults):

# In config/runtime.exs, add:

config :cargo_elixir, :fleet,

mqtt_broker: System.get_env("FLEET_MQTT_BROKER", "mqtt.buoy.fish"),

mqtt_port: String.to_integer(System.get_env("FLEET_MQTT_PORT", "1883")),

mqtt_user: System.get_env("FLEET_MQTT_USER", "gateway"),

mqtt_pass: System.get_env("FLEET_MQTT_PASS", "buoy")

# 6. Integration with Existing Code

This section covers how fleet management connects to modules that already exist in CargoElixir. The goal is to extend, not duplicate.

## 6.1 Gateway Schema Extension

The existing CargoElixir.Gateways.Gateway schema already has role, state, and location fields. Add fleet-specific fields via a migration:

# New migration: add_fleet_fields_to_gateways.exs

defmodule CargoElixir.Repo.Migrations.AddFleetFieldsToGateways do

use Ecto.Migration

def change do

alter table(:gateways) do

add :companion_device_id, :string    # Links to fleet device_id

add :tailscale_ip, :string

add :fleet_online, :boolean, default: false

add :fleet_last_seen_at, :utc_datetime

end

create index(:gateways, [:companion_device_id])

end

end

Then update the Gateway changeset to cast the new fields. This lets the fleet dashboard show which gateway each Pi companion is attached to.

## 6.2 ChirpStack Client (DO NOT REBUILD)

CRITICAL: The app already has a complete ChirpStack gRPC client. Do NOT create a new REST client with {:req, ...}. Use the existing modules:

| Module | Purpose |
| --- | --- |
| CargoElixir.Chirpstack.Client | High-level abstraction. Routes to gRPC or REST based on CHIRPSTACK_USE_GRPC env var. |
| CargoElixir.Chirpstack.GrpcClient | Direct gRPC calls. Has create_gateway, get_gateway, delete_gateway, and get_gateways_batch. |
| CargoElixir.Chirpstack.Connection | Channel management. get_channel_with_metadata/0, auth_metadata/0, tenant_id/0. Config from PlatformSettings DB or env vars. |
| CargoElixir.Gateways | Context module. create_gateway/1, sync_all_gateways/0, retry_chirpstack_sync/1. Already handles local DB + ChirpStack sync. |
| CargoElixirWeb.GatewayController | REST API. Full CRUD + sync_status, sync_all, list_chirpstack_gateways actions. |

## 6.3 Batch Operation Pattern

CRITICAL: Never loop over single-operation gRPC functions. Each call to get_gateway/1 opens and closes a gRPC connection. For bulk operations, use get_gateways_batch/1 or write a new _batch function that opens one connection and iterates. See CLAUDE.md in the repo root for the full pattern.

# WRONG - opens N connections for N gateways:

Enum.each(euis, fn eui -> GrpcClient.get_gateway(eui) end)

# RIGHT - opens 1 connection for N gateways:

GrpcClient.get_gateways_batch(euis)

## 6.4 Auth Pipeline

Fleet API endpoints should use the existing auth pipeline. For the initial rollout, use the platform_admins scope (founding team only). The auth flow is: React sends Supabase JWT in Authorization header, AuthPlug verifies via JWKS, then scope middleware checks role.

# 7. Data Flow Diagrams

## 7.1 LoRa Packet Flow

Each Pi companion is also a Helium node. Packets flow through two paths simultaneously:

LoRa Sensors

|

| (LoRa radio)

v

RAK2287 Concentrator (on Pi HAT)

|

| (SPI)

v

udp-packet-forwarder --(UDP 1700)--> cs-mux --> gateway-rs (Helium)

|               |

|               v

|         Helium Packet Router

|               |

|               v

+---------> ChirpStack (your server)

|

v

POST /api/payloads

|

v

CargoElixir.Payloads

|

v

broadcast("payload:org:{slug}")

## 7.2 Fleet Management Flow

Pi Agent (buoy_agent.py)

|

| MQTT (buoy/fleet/{id}/status)

v

mqtt.buoy.fish:1883

|

v

CargoElixir.Fleet.MqttClient (GenServer)

|

v

CargoElixir.Fleet.MqttHandler

|

+-- update_status --> DeviceRegistry (ETS)

|

+-- Endpoint.broadcast("fleet:status", "device_status", payload)

|

v

FleetChannel (WebSocket)

|

v

React Frontend

## 7.3 Command Flow (React to Pi)

React UI

|

| WebSocket: handle_in("send_command", ...)

| or POST /api/admin/fleet/devices/:id/command

v

FleetController / FleetChannel

|

v

CargoElixir.Fleet.MqttClient.send_command/3

|

| MQTT publish (buoy/fleet/{id}/commands)

v

Pi Agent --> executes --> publishes ack

|

| MQTT (buoy/fleet/{id}/commands/ack)

v

MqttHandler --> CommandTracker.resolve/2

|

+-- Endpoint.broadcast("fleet:commands:{id}", "command_ack", result)

|

v

React UI (updates command status)

# 8. New File Layout

All new files follow existing CargoElixir conventions (snake_case, context modules, controller separation):

lib/

cargo_elixir/

fleet/

mqtt_client.ex          # GenServer - MQTT connection + publish

mqtt_handler.ex         # Tortoise handler - routes messages

device_registry.ex      # ETS-backed device state

command_tracker.ex      # In-flight command tracking

cargo_elixir_web/

channels/

fleet_channel.ex        # WebSocket channel for React

controllers/

fleet_controller.ex     # REST API for fleet operations

priv/repo/migrations/

YYYYMMDDHHMMSS_add_fleet_fields_to_gateways.exs

config/

runtime.exs                 # Add :fleet config block

# 9. Testing Strategy

## 9.1 Unit Tests

test/

cargo_elixir/

fleet/

device_registry_test.exs    # ETS operations, cleanup timer

command_tracker_test.exs    # Track/resolve/timeout

mqtt_handler_test.exs       # Message routing, JSON parsing

cargo_elixir_web/

channels/

fleet_channel_test.exs      # Join auth, push events

controllers/

fleet_controller_test.exs   # REST endpoints

## 9.2 Integration Testing with MQTT

For local dev, use the existing Mosquitto broker or start one in Docker. The MqttClient GenServer can be configured with test credentials:

# In config/test.exs:

config :cargo_elixir, :fleet,

mqtt_broker: "localhost",

mqtt_port: 1883,

mqtt_user: "test",

mqtt_pass: "test"

## 9.3 Manual Smoke Test

After implementation, verify end-to-end with mosquitto_pub:

# Simulate a heartbeat from a Pi

mosquitto_pub -h mqtt.buoy.fish -u gateway -P buoy \

-t "buoy/fleet/baja-companion-1/status" \

-m '{"device_id":"baja-companion-1","site_name":"Baja Sur",

"tailscale_ip":"100.64.1.5",

"system":{"cpu_percent":12,"memory_percent":45,"cpu_temp":52},

"containers":[{"name":"udp-pf","status":"running"},

{"name":"cs-mux","status":"running"},

{"name":"gateway-rs","status":"running"},

{"name":"buoy-agent","status":"running"}]}'

# Then check the React frontend:

# 1. Open app.buoy.fish, navigate to fleet dashboard

# 2. Verify baja-companion-1 appears as online

# 3. Click "restart container" button

# 4. Check MQTT for the command on buoy/fleet/baja-companion-1/commands

# 10. Implementation Phases

## Phase 1: MQTT Core (1-2 days)

Add tortoise311 dependency. Create Fleet.MqttClient, Fleet.MqttHandler, Fleet.DeviceRegistry, and Fleet.CommandTracker. Add to supervision tree. Verify with mosquitto_pub that heartbeats update ETS and PubSub broadcasts fire.

## Phase 2: REST API + WebSocket Channel (1-2 days)

Create FleetController with index/show/send_command/generate_site_config. Create FleetChannel following PayloadChannel pattern. Register in UserSocket. Add routes to router under platform_admins scope. Test with the React dev environment.

## Phase 3: Gateway Schema Extension (1 day)

Add migration for companion_device_id, tailscale_ip, fleet_online, fleet_last_seen_at. Update Gateway changeset. Wire DeviceRegistry updates to also update the Gateway record when a companion_device_id matches.

## Phase 4: React Frontend (2-3 days)

Build fleet dashboard page in the existing React app. Connect to FleetChannel WebSocket for real-time updates. Implement device cards, command buttons, log viewer. Follow existing React component patterns in the codebase.

## Phase 5: Testing and Deployment (1-2 days)

Write unit tests for all Fleet modules. Integration test with local Mosquitto. Deploy to staging with a real Pi companion running the buoy-gateway-image. Verify end-to-end: Pi agent heartbeat appears in React dashboard, commands execute on Pi.

## Key Reminders for the Developer

| 1. | This is CargoElixir, not Buoy. Module names are CargoElixir.Fleet.*, controllers are CargoElixirWeb.*. |
| --- | --- |
| 2. | No LiveView. The frontend is React 18.2. All real-time goes through WebSocket channels (FleetChannel, same pattern as PayloadChannel). |
| 3. | Do NOT build a new ChirpStack REST client. Use the existing gRPC client (CargoElixir.Chirpstack.GrpcClient) and context (CargoElixir.Gateways). |
| 4. | Never loop single gRPC calls. Use _batch functions. Read CLAUDE.md in the repo root. |
| 5. | Auth is Supabase JWT + JWKS. Use existing auth pipeline in router.ex. Fleet endpoints go in platform_admins scope initially. |
| 6. | The only new dependency is {:tortoise311, "~> 0.12"}. Everything else is already in mix.exs. |
| 7. | Pi companions are also Helium nodes. The UDP PF must remain in the Docker stack. Packets flow through both Helium Packet Router and ChirpStack. |